# Tab 1

SIH CALCULATION & MODEL JUSTIFICATION REPORT

Closed-Container Dataset

1. Summary

The workbook implements a complete calculation. The measured signal starts as a 10-bit ADC(Analog to Digital converter) count, is converted to the voltage across a load resistor, then to MQ135 sensor resistance, normalized as Rs/R0, converted through an empirical power-law calibration to a gas-concentration proxy, and finally mapped to an estimated vitamin-C value. A separate first-order degradation model provides a time-based reference value. The two vitamin-C estimates are compared through a percentage-error calculation. The Temp sheet independently models cooling and humidity with exponential first-order responses.

2. Data Architecture and Why Each Stage Exists

| Stage | Input | Equation / operation | Purpose |
| --- | --- | --- | --- |
| 1. Measurement | Raw ADC count | ADC count is retained from the MQ135 gas sensor. | Direct digital representation of sensor voltage |
| 2. Electrical conversion | ADC → voltage | V = ADC/(Max. ADC) * Vref | Recover analog voltage represented by the ADC |
| 3. Sensor resistance | Voltage → Rs | Rs = ((Vref−V)/V)RL | Convert divider voltage into MQ135 resistance |
| 4. Normalization | Rs and R0 | Rs/R0 | Remove absolute resistance scale for gas-sensor calibration |
| 5. Ethylene Gas concentration (Ce, in ppm) | Rs/R0 | C = a * (Rs/R0) ^ b | Empirical power-law calibration |
| 6. Theoretical vitamin concentration (Cv, in ppm) | Time | C0 * exp(−kt) | First-order retention/degradation reference |
| 7. Calculated vitamin concentration (cv, in ppm) | Cv, Ce | C0 − m ln(Ce+1) | Empirical mapping used by the prototype |
| 8. Error | Two vitamin-C estimates | ¦Cv - cv¦/Cv ×100 | Quantifies disagreement between the two model outputs |

3. Electrical Measurement Calculations

3.1 ADC count to voltage

Implemented equation:

V = (ADC / 1023) × 5

Here

1023 = 2^10−1, the sensor and microcontroller is a 10-bit ADC.

5 = V_ref, the voltage from the Arduino UNO board.

Example at t = 0 s: ADC = 251 → V = 251/1023 × 5 =  1.226783969.

3.2 Load-resistor / MQ135 resistance calculation

Implemented equation:

Rs = ((5 − V) / V) × 10000

Here

5 = V_ref, the voltage from the Arduino UNO board.

10000 = load resistance with units Ω.

Example at t = 0 s, Rs = ((5−1.226783969)/1.226783969)×10 = 30756.97211.

3.3 Normalized sensor ratio

Implemented equation:

Ratio = Rs / Ro

Here

3.31 R0 Calibration

The Rs(clean)=98.3 kΩ and Rs/R0=3.6. Therefore:

Ro = 98.3 / 3.6 = 27.305kΩ

Ro = 27.305

Normalizing Rs by R0 is conventional for metal-oxide gas-sensor calibration because the sensor response is commonly represented as a resistance ratio.

4. MQ135 Gas-Concentration Model

Implemented  equation:

C_VOC = a × (Rs/R0)^b

4.1 Ethylene Power-Law Calibration

From the experimental data of two ethylene calibration points: 20 ppm at Rs/R0=0.65 and 100 ppm at Rs/R0=0.35.

So, 20 = a * (0.65)^b  and  100 = a * (0.35)^b

Therefore the equation is:

Cethylene = 6.53 (Rs/R0)^−2.6  ppm

This is a calibrated curve for the sensor. It is not a universal MQ135 equation. Power-law behavior is commonly used to represent metal-oxide gas-sensor response over an appropriate calibration range.

The negative power means that an increasing normalized resistance produces a decreasing calculated concentration. This is consistent with the general use of Rs/R0 response curves for MQ-family sensors, but the numerical constants a and b are calibration parameters, not universal constants of the MQ135.

Example at t = 0 s: Rs/R0 = 30.75697/40 = 0.768924.

Therefore C = 116.6(0.768924)^−2.769 ≈ 241.37 ppm

5. First-Order Vitamin-C Retention Model

Implemented equation:

Cref(t) = Co × exp[−k × (t/86400)]

Here

Co = 103 ppm

k = 0.045 day⁻¹

t = time in seconds

This is the exponential solution of a first-order degradation model dC/dt = −kC. First-order kinetics are widely used to model ascorbic-acid/vitamin-C degradation under specified storage conditions.

Example at t = 0: Cref = 103.00. At t = 86400 s (1 day): C_ref = 103e^−0.045 = 98.47. The corresponding one-day retention is about 95.6% of the initial model value.

6. Derivation of the Gas-Sensor Vitamin-C Equation

In plant biology, degradation of organic compounds (like Vitamins) often follow an exponential (or) log decay pattern.

Since, the food can only decay, meaning the concentration of Ce strictly increases at the time increases, therefore we can relate concentration of vitamins with concentration of ethylene as,

Cv = 103 − m * ln(Ce + 1)

The +1 prevents ln(0). This equation is not a fundamental chemical equation, but a calibrated gas-response proxy to the vitamin-C reference scale.

Here	Ce = concentration of ethylene which changes with time t.

m = assumed to be constant, neglecting minor effects.

For each sample, the local coefficient is calculated as:

m = [103 − Cref,i] / ln(Cethylene + 1)

From experimenting with different values of theoretical vitamin concentration 		and actual concentration, we get a value of m. The average of m is

0.3900579357.

The final equation :

Cv = 103 − 0.3900579357 ln(Ce + 1)

7. Dataset Statistics

| Metric | Result |
| --- | --- |
| Number of samples | 86400 |
| ADC | 246 to 953; mean 599.486 |
| V_RL | 1.202 to 4.658 V |
| R_s | 0.735 to 31.585 kΩ |
| Ethylene proxy | 4.472 to 78982.248 ppm |
| C_ref | 98.468 to 103.000 |
| C_sensor | 98.601 to 102.337 |
| Signed mean error | 0.0565 % |
| Mean absolute error | 0.3131 % |
| RMSE of error | 0.3550 % |

The signed mean error is near zero because positive and negative errors cancel. For an accuracy statement, mean absolute error is the clearer metric: 0.313% for this dataset.

8. Thermal Model

8.1 Thermal Mass & Effective Conductance

The total thermal capacitance (C_th) is calculated by summing the heat capacities of the internal air volume, the primary aluminum heat sink, and the structural enclosure walls:

• Air Thermal Capacity: C_air = ρ × V × C_v = 1.2 kg/m³ × 0.03 m³ × 718 J/(kg·K) =

25.85 J/K

• Heat Sink Capacity: C_sink = m × C_p = 0.15 kg × 900 J/(kg·K) = 135.00 J/K

• Wall Structural Capacity: C_wall = 40.00 J/K

• Total Thermal Capacitance: C_th = 25.85 + 135.00 + 40.00 = 200.85 J/K

The effective heat-transfer conductance (G) represents enclosure losses to ambient environment for a given temperature differential. With surface area A = 0.59 m² and total thermal resistance R_total = 0.2098 K/W:

G = A / R_total = 0.59 / 0.2098 = 2.8122 W/K

8.2 TEC Steady-State Temperature

The steady-state energy balance for the thermoelectric cooler (TEC) incorporates enclosure losses, Joule heating, and conduction across the TEC elements:

T_min = [G × T_out + (1/2) × I²R + K × T_h] / [G + αI + K]

Given parameters: T_out = 303.15 K, T_h = 318.15 K, Seebeck coefficient α = 0.051 V/K, drive current I = 4.2 A, internal resistance R = 2.2 Ω, thermal conductance K = 0.6 W/K, and enclosure conductance G = 2.8122 W/K.

Substituting these values yields:

T_min = [(2.8122)(303.15) + 0.5(4.2²)(2.2) + (0.6)(318.15)] / [2.8122 + (0.051)(4.2) + 0.6] = 293.076 K (19.93 °C)

8.3 Derivation of the Thermal Time Constant

For a lumped thermal system, the system time constant τ is the ratio of thermal capacitance to total effective thermal conductance (G_eff):

G_eff = G + αI + K = 2.8122 + (0.051)(4.2) + 0.6 = 3.6264 W/K

τ = C_th / G_eff = 200.85 / 3.6264 = 55.3855 s

9. Temperature Model

T(t) = T∞+(T0−T∞)e^−t/τ

Here

T∞ = 293.076 K

T0 = 305.632 K

τ = 55.3855 s

10. Scientific References

Spinelle, L., Gerboles, M., Kok, G., Persijn, S., & Sauerwald, T. (2017). Review of portable and low-cost sensors for ambient air monitoring of benzene and other VOCs. Sensors, 17(7), 1520. https://www.mdpi.com/1424-8220/17/7/1520

Ahlers, S., Müller, G., & Doll, T. (2005). A rate equation approach to the gas sensitivity of thin film metal oxide materials. Sensors and Actuators B: Chemical. https://www.sciencedirect.com/science/article/pii/S0925400504007580

Schultealbert, C., Baur, T., Schütze, A., Böttcher, S., et al. (2017). A novel approach towards calibrated measurement of trace gases using metal oxide semiconductor sensors. Sensors and Actuators B: Chemical. https://www.sciencedirect.com/science/article/pii/S0925400516312278

Muppidathi, B. V., Ngoune, B. B., Hallil, H., et al. (2024). Assessment of standardization-based calibration transfer techniques for gas sensors. IEEE Sensors. https://ieeexplore.ieee.org/abstract/document/10700614/

Peleg, M. (2018). Modeling the degradation kinetics of ascorbic acid. Critical Reviews in Food Science and Nutrition. https://www.tandfonline.com/doi/abs/10.1080/10408398.2016.1264360

Peleg, M. (2017). Theoretical study of aerobic vitamin C loss kinetics during commercial heat preservation and storage. Food Research International. https://www.sciencedirect.com/science/article/pii/S0963996917306932

Bosch, V., Cilla, A., García-Llatas, G., Gilabert, V., et al. (2013). Kinetics of ascorbic acid degradation in fruit-based infant foods during storage. Journal of Food Engineering. https://www.sciencedirect.com/science/article/pii/S0260877412005870

Remini, H., Mertz, C., Belbahi, N., Achir, N., Dornier, M., et al. (2015). Degradation kinetic modelling of ascorbic acid and colour intensity in pasteurised blood orange juice during storage. Food Chemistry. https://www.sciencedirect.com/science/article/pii/S0308814614016318

Tiwari, B. K., O'Donnell, C. P., & Muthukumarappan, K. (2009). Ascorbic acid degradation kinetics of sonicated orange juice during storage. LWT - Food Science and Technology. https://www.sciencedirect.com/science/article/pii/S0023643808002594

Giannakourou, M. C., & Taoukis, P. S. (2021). Effect of alternative preservation steps and storage on vitamin C stability: critical review and kinetic modelling approaches. Foods, 10(11), 2630. https://www.mdpi.com/2304-8158/10/11/2630